Hard Disk Drive Read Header Controller Design Project

EE 141 – Principles of Feedback Control

Spring 2015

University of California, Los Angeles

# Baixiao Huang  504313981

# Weiqian Xu    	404297854

# Xingjian Yan  	104304403

05/21/2015

1.0  OBJECTIVES

# The objective of this project is to design a control system for hard disk drive read header. Knowledge of feedback control theories and skills of using Matlab are crucial in designing. The system contains a controller, a motor coil, a reader arm and a sensor forming a closed loop system. In this project, we are going to analyze the relationship between system performance and different compensators or sensors.

# Figure 1

# 2.0  TASKS

# 2.1	 Task 1

# For this part, we are provided with the physical model relating the input torque u to the header position y with the inertia of arm and head J and friction b.

# Jy+by=u(t)                               		(1)

# Simply apply Laplace transform to get the transfer function:
                                 G2s=Y(s)U(s)= 1Js2+bs             					(2)

# 2.2	 Task 2

# The transfer function of the motor coil that relates the input voltage to the output torque is related by:

# G1s= U(s)V(s)= KmLs+R                          		(3)

# In this task, we are going to obtain the open-loop transfer function of the cascaded HDD head reader assembly and then use Matlab to generate the plot of the system’s step response.

# G1sG2s= KmLs+R*(Js2+bs)= KmLJs3+Lbs2+RJs2+Rbs           		   	(4)

# The Matlab code with the constants plugged in and the plot obtained are presented below:

s = tf('s');

J = 1;

b = 20;

R = 1;

L = 0.001;

Km = 5;

G1 = Km/(L*s+R)

G2 = 1/(J*s^2+b*s)

openTF = G1*G2

t=[0:0.005:0.5];

y = step(openTF,t);

plot(t,y);

# Figure 2

# As we can see, if a constant voltage is applied, then the read head will moves with a constant speed. However, at the beginning when the voltage is applied, there is a curvature indicating the read head is accelerating.

# 2.3	 Task 3 – Proportional Compensator

# In this part, we are going to make the closed-loop system satisfies several transient response performance specifications: Percent overshoot less than 5%; Settling time (2% deviation) less than 250ms; Maximum value of response to a unit step disturbance less than 5×10-3. Assume the feedback sensor’s transfer function is H(s) = 1 and the candidate controller is Fs= Ka.

# Part A

# We are going to determine and evaluate the responses of the closed-loop system to the unit step reference input without disturbance. If the proportional compensator applied, the transfer function is given by:

# KaG1(s)G2(s)                 					(5)

# Then the closed-loop transfer function is given by:

# KaG1(s)G2(s)1+KaG1(s)G2(s)                                  	(6)

# After plugging in (2) and (3), the equation becomes:

# KaKmLJs3+Lb+RJs2+Rbs+KaKm      							(7)

# Try Ka=50, the Matlab code (continuous after the previous code) and plot are shown below:

Ka = 50;

t=[0:0.005:1.5];

ProportionalTF =(Ka*openTF)/(1+Ka*openTF)

y = step(ProportionalTF, t);

plot(t,y);

# Figure 3

# If Ka=400, the Matlab code and plot are shown below:

Ka = 400;

ProportionalTF = (Ka*openTF)/(1+Ka*openTF)

y = step(ProportionalTF, t);

plot(t,y);

# Figure 4

# It is clear that the system overshoot when Ka=50 is much lesser than that when Ka=400. If the feedback amplification is too big, the system will be too sensitive.

# Part B

# In this part, we are going to determine and evaluate the responses of the closed-loop system to the unit step disturbance, assuming zero reference input. The transfer function for disturbance is:

# Tω= G2(s)1+KaG1(s)            						(8)

# As above, we first plot the response with Ka=50.

Ka = 50;

Tw = G2/(1+Ka*G1*G2)

y = step(Tw, t);

plot(t,y);

# Figure 5

# Similarly, we then try Ka=400.

Ka = 400;

Tw = G2/(1+Ka*G1*G2)

y = step(Tw, t);

plot(t,y);

# Figure 6

# Same as Part A, Ka=50 has less overshoot than Ka=400.

# Part C

# In this part, the objective is to find a value for Ka that satisfies the performance specifications above. We can plot overshoot percentage and settling time as a function of Ka, and analyze the optimal Ka for the system.

overshoot = [];

settlingTime = [];

maxError = [];

n=[1:100];

for  Ka=n

ProportionalTF = (Ka*openTF)/(1+Ka*openTF);

y = step(ProportionalTF, t);

info = stepinfo(y, t, 'SettlingTimeThreshold', 0.02);

overshoot = [overshoot, info.Overshoot];

settlingTime = [settlingTime info.SettlingTime];

Tw = G2/(1+Ka*G1*G2);

y = step(Tw, t);

maxError = [maxError max(y)];

end

subplot(3,1,1);

plot(n,overshoot);xlabel('Ka');ylabel('Overshoot Percentage');

subplot(3,1,2);

plot(n,settlingTime);xlabel('Ka');ylabel('Settling Time');

subplot(3,1,3);

plot(n,maxError);xlabel('Ka');ylabel('Maximum Error of Disturbance');

# Figure 7

# For overshoot less than 5%, Ka is required to be equal or less than 41. The value of Ka that satisfies settling time requirement is more than 100 and is not in the graph. For disturbance less than 0.005, Ka is required to be equal or bigger than 41. As a result, the three requirements cannot be satisfied at the same time.

# 2.4	 Task 4 – Positional and Velocity Sensor

# In this part, we substitute the sensor with Hs=1+ KHs that measures the position as well as the velocity of the head reader. Then we need to find Ka and KH such that the system is stable and satisfies the performance specifications mentioned above. The closed loop transfer function is:

# KaG1(s)G2(s)1+KaHsG1(s)G2(s)                   				(9)

# The fundamental idea for finding the solution is that since both Ka and Kh are varying, we can plot a 3-D graph in which x-axis is Ka, y-axis is Kh, and z-axis is the property under examination including settling time, overshoot and disturbance. Then we can find out all of the combinations of Ka and Kh that satisfy all performance requirements.

[KaRange, KhRange] = meshgrid(55:65, 0:0.01:0.1);

overshootMatrix = [];

settlingTimeMatrix = [];

candidatePairs = [];

t=[0:0.05:0.8];

G12= G1*G2;

for Kh = KhRange(:,1)'

%overshootArr=[];

%settlingTimeArr=[];

for Ka = KaRange(1,:)

CLTF = (Ka*G12)/(1+Ka*(1+Kh*s)*G12);

y = step(CLTF, t);

info = stepinfo(y, t, 'SettlingTimeThreshold', 0.02);

%overshootArr = [overshootArr, info.Overshoot];

%settlingTimeArr = [settlingTimeArr, info.SettlingTime];

Tw = G2/(1+Ka*(1+Kh*s)*G12);

y = step(Tw,t);

maxDisturbance = max(y);

if(info.Overshoot <= 5 & info.SettlingTime <0.25 & y<0.005)

candidatePairs = [candidatePairs; Ka, Kh];

end

end

%overshootMatrix = [overshootMatrix; overshootArr];

%settlingTimeMatrix = [settlingTimeMatrix; settlingTimeArr];

end

%mesh(KaRange, KhRange, settlingTimeMatrix);

%mesh(KaRange, KhRange, overshootMatrix);

candidatePairs

candidatePairs =

56.0000    0.0300

57.0000    0.0300

58.0000    0.0300

59.0000    0.0300

60.0000    0.0300

61.0000    0.0300

62.0000    0.0300

63.0000    0.0300

64.0000    0.0300

65.0000    0.0300

# There are a bunch of valid pairs of Ka and Kh that satisfies the design. We can select Ka=60 and Kh =0.03 to check the result.

Ka = 60; Kh = 0.03;

CLTF = (Ka*G12)/(1+Ka*(1+Kh*s)*G12);

y = step(CLTF, t);

figure(6);

plot(t,y);title('System Step Response');

info = stepinfo(y, t, 'SettlingTimeThreshold', 0.02)

Tw = G2/(1+Ka*(1+Kh*s)*G12);

y = step(Tw,t);

figure(7);

plot(t,y);title('System Step Disturbance Response');

maxDisturbance = max(y)

info =

RiseTime: 0.1621

SettlingTime: 0.2380

SettlingMin: 0.9401

SettlingMax: 1.0084

Overshoot: 0.8390

Undershoot: 0

Peak: 1.0084

PeakTime: 0.3500

maxDisturbance = 0.0034

# The graphs of responses are presented below.

# Figure 8

# Figure 9

# We can then conclude that the pairs of results listed above are valid for the system to meet all requirements.

# 2.5	 Task 5 – PID Compensator

# In this part, we revert the sensor back to H(s) = 1 but substitute the controller with Fs= K1+ K2s+ K3s. Since the arm inherently has an integrating term, we assume K2=0. The closed-loop transfer function then becomes:

# (K1+K3s)G1(s)G2(s)1+(K1+K3s)G1(s)G2(s)        					   (10)

# We can apply the same procedure to generate 3-D graph of constraints.

# Figure 10 – Settling time

# When K1 is around 100 and K3 is around 5 to 10, the settling time requirement can be satisfied. With the range of values, we are able to further derive detailed solution. If we take a closer look at the range of values that satisfy the constraints, the graph becomes the one shown by Figure 11.

# Figure 11 – Settling time

# As shown in Figure 11, there are couples of K1 and K3 pairs that meet the settling time requirement. If we exam K1=100 and K3=5 for example, the settling time will under 250ms. The next step is to evaluate the overshoot of the system.

# Figure 12 – Overshoot

# As we can see, for K1=100 and K3=5, there is no apparent overshoot. We can then examine the disturbance of this range.

# Figure 13 – Disturbance

# The disturbance of K1=95 and K3=100 meets the requirement. The code for generates graphs above are presented below:

[K1Range K3Range] = meshgrid(95:105, 5:10);

overshootMatrix = [];

settlingTimeMatrix = [];

disturbanceMatrix=[];

candidatePairs = [];

t=[0:0.2:15];

G12= G1*G2;

for K3 = K3Range(:,1)'

overshootArr=[];

settlingTimeArr=[];

disturbanceArr=[];

- for K1 = K1Range(1,:)

- CLTF = ((K1+K3*s)*G12)/(1+(K1+K3*s)*G12);

- y = step(CLTF, t);

- info = stepinfo(y, t, 'SettlingTimeThreshold', 0.02);

- overshootArr = [overshootArr, info.Overshoot];

- settlingTimeArr = [settlingTimeArr, info.SettlingTime];

- Tw = G2/(1+(K1+K3*s)*G12);

- y = step(Tw,t);

- maxDisturbance = max(y);

- disturbanceArr = [disturbanceArr, maxDisturbance];

      - if(info.Overshoot <= 5 & info.SettlingTime <0.25 & y<0.005)

- candidatePairs = [candidatePairs; Ka, Kh];

              - end

end

- overshootMatrix = [overshootMatrix; overshootArr];

- settlingTimeMatrix = [settlingTimeMatrix; settlingTimeArr];

- disturbanceMatrix = [disturbanceMatrix; disturbanceArr];

- end

- mesh(K1Range, K3Range,

overshootMatrix);xlabel('K1');ylabel('K3');zlabel('Overshoot')

- mesh(K1Range, K3Range,

settlingTimeMatrix);xlabel('K1');ylabel('K3');zlabel('SettlingTime')

- mesh(K1Range,

- K3Range,disturbanceMatrix);xlabel('K1');ylabel('K3');zlabel('Disturbance');view(3

- candidatePairs

To examine the correctness of the answer, we select K1=100 and K3=5 to evaluate more specifically

- K1=100; K3=5;

- CLTF = ((K1+K3*s)*G12)/(1+(K1+K3*s)*G12);

- y = step(CLTF, t);

- info = stepinfo(y, t, 'SettlingTimeThreshold', 0.02)

- plot(t,y);

- info =     RiseTime: 0.0879

- SettlingTime: 0.1560

- SettlingMin: 0.9211

- SettlingMax: 1.0000

- Overshoot: 0

- Undershoot: 0

- Peak: 1.0000

- PeakTime: 0.8000

# Figure 14 – System overshoot

# With the information generated and the curve shown in Figure 14, we can discover that at the same time satisfying other system requirements, the system has no overshoot. The reason is that K3 acts as a damping factor that prevents the response from overshooting. We can then analyze the disturbance.

# Figure 15 – System disturbance

# The maximum disturbance only reaches 2 × 10-3, within the range of requirement. We can conclude for this task that, compared to the proportional compensator, the PID compensator can be much more effective in controlling the system especially reducing the overshoot.

# 2.6	 Task 6 – PID Compensator

# In this part, we add a spring load on the arm; in the system diagram is anther block G3, with

# transfer function					 G3= ωnS2+2ZωnS+ ωn2 , Z=0.3   ωn=18850 rad/s

# The transfer function of the closed loop system is:

# T=FG1G2G31+HFG1G2G3

# We use the same PD compensator as in part 5 for F and keep H = 1.

# The block diagram is shown below.

Actual Head

Position

Spring Load

G3(s)

Arm

G2(s)

Motor Coil

G1(s)

Controller F(s)

# Desired

# Head

# Position

Sensor

H(s)

# Part A

# The step response of the spring load system G3, is shown in figure 16.

# Figure 16 – unit step response of spring load

# Part B

# Using similar method as in part 5, we use a double loop to find out the relation between K1 and K3, as show in figure 17, and the candidate pairs (of K1 and K3).

# Figure 17 – Overshoot and settling time relation with both K1 and K3

# Using this method we were able to find a candidate pair that satisfies the requirement:

# K1 = 45    K3 =0.8

# The performance of the closed loop system with this specific pair is:

RiseTime: 0.0173

SettlingTime: 0.0262

SettlingMin: 0.9356

SettlingMax: 1.0082

Overshoot: 0.8169

Undershoot: 0

Peak: 1.0082

PeakTime: 0.0400

# As the numbers show, in terms of performance, we are way above the requirement. A step response diagram is shown in figure 18.

# Figure 18 – step response of tuned closed loop system

# The 3dB bandwidth of the system is 13.6 rad/s

# Whereas in the Bode plot in figure 19 shows that the resonant peak, appears to be at 10000 rad/s.

# Figure 19 – bode plot shows that that there is a resonant peak at about 1000 rad/s

# Part C

GainMargin =

6.6390e+03

PhaseMargin =

-180

# 3.0  CONCLUTION

# We find the system using a PD controller and spring load to be most satisfying.

# Here’s the block diagram again:

Actual Head

Position

Spring Load

G3(s)

Arm

G2(s)

Motor Coil

G1(s)

Controller F(s)

# Desired

# Head

# Position

Sensor

H(s)

# Transfer function of each block are presented below:

# Fs=K1+ K2s+ K3s   						    (11)

# G1s= KmLs+R     								(12)

# G2s= 1Js2+bs          					    (13)

# G3s= ωn2s2+2ζωns+ωn2  = 11+2ζsωn+s2ωn2                         	(14)

# Hs= 1                        			 	(15)

# The closed-loop transfer function for the entire system is:

# F(s)G1(s)G2(s)G3(s)1+F(s)G1(s)G2(s)G3(s)  					  			(16)

# Parameters are shown in the table below:

# Table 1

| # Parameters | # Value |
| --- | --- |
| # Inertia of arm and head J # Friction b # Field resistance R # Field inductance L # Motor constant Km # ζ # ωn # K1 # K2 # K3 | # 1 N∙m∙s2/rad # 20 Kg/m/s # 1 Ω # 0.001 H # 5 N∙m/A # 0.3 # 18850 rad # 0 |

# We compared the system performance with the given specifications in Table 2

# Table 2

| # Specification Variable Name | # Specification Variable Value | # Designed System Value |
| --- | --- | --- |
| # Percent Overshoot # Settling time(2% deviation) # Maximum value of response to a unit step disturbance | # < 5% # < 250ms # < 5 × 103 | # 0.8% # 26ms |